LIGO Laboratory / LIGO Scientific Collaboration

LIGO-T1000319-v1	LIGO	20th June 2010

Note on Choice of Lowest Stage Wire Radius for Quad Reaction Chain

Norna A Robertson

Distribution of this document:

LIGO Scientific Collaboration

This is an internal working note

of the LIGO Laboratory.

| California Institute of Technology LIGO Project – MS 18-34 1200 E. California Blvd. Pasadena, CA 91125 Phone (626) 395-2129 Fax (626) 304-9834 E-mail: info@ligo.caltech.edu | Massachusetts Institute of Technology LIGO Project – NW22-295 185 Albany St Cambridge, MA 02139 Phone (617) 253-4824 Fax (617) 253-7014 E-mail: info@ligo.mit.edu |
| --- | --- |
| LIGO Hanford Observatory P.O. Box 159 Richland WA 99352 Phone 509-372-8106 Fax 509-372-8137 | LIGO Livingston Observatory P.O. Box 940 Livingston, LA 70754 Phone 225-686-3100 Fax 225-686-7189 |

http://www.ligo.caltech.edu/

# Introduction

The purpose of this document is to capture the final design choice for the wire radius for the final stage of the reaction chains for the ETM and ITM suspensions.

# Background

The choice of wire radii for Advanced LIGO suspensions has in general been made by using a nominal value for tensile strength for music wire of 2000 MPa and using a safety factor of  ~3. We note that the minimum tensile strength as per ASTM A-228M-07 is ~2000 MPa for wire of diameter ~1.5 mm, radius 0.75 mm, and gradually increases for smaller diameters (see T0900502). The range of wire radii for a quad is from ~0.5 mm at the top to ~ 0.23 mm at the bottom and for these values the minimum tensile strength is of order 2400 to 2600 MPa, so we have in fact a safety factor closer to 4.

The quadruple pendulums are all assembled firstly with metal masses and wires at every stage. Thus we have to specify a wire radius for the bottom stage of the main chain, though this gets replaced eventually by the silica fibres when the monolithic suspension is installed. This wire is chosen using the criteria above (stress of ~1/3 of 2000 MPa) applied to a 40 kg mass and the resulting value is 0.457 mm diameter, 0.2285 mm radius, ref D060516-v2. However the bottom mass of the reaction chain is lighter, 26 kg in the case of the end reaction mass for the ETM, and 20 kg for the newly proposed thinner compensator plate in the reaction chain of the ITM. If one follows the same criteria for choosing those wires, the wires for suspending those masses would be thinner than for the main chain, and indeed different for the two types of reaction chain. We had originally proposed to use thinner wire for the reaction chain – see T060283-02. However for simplicity of assembly we are now proposing that the same wire diameter is used for all chains at the final stage of the quad suspensions, regardless of the final mass. We review the implications of this decision in section 4.

Note that choice of wire diameters at upper stages is not affected by any difference in the lowest mass since we keep the overall mass the same by increasing the penultimate mass as necessary.

# Parameters varied for this study.

We have looked at the effect of different wire diameters using the latest published quad MATLAB model from Mark Barton, ref T1000263-v1, which is the current baseline model of the quad suspension main chain. To used this model for looking at the reaction chain we require to change several of the parameters

1) Material (Young’s modulus) and radius of the bottom wire.

2) Masses and moments of inertia for the penultimate and bottom mass.

3) The effective d values at the ends of the lowest wire, to simulate keeping the physical d values (i.e. positions of attachment point for the break off prisms or clamps) the same. The effective d values will change with wire diameter since they include the wire flexure lengths, and the code is currently written so that if a wire is changed, the physical d is changed to keep the effective d constant. Thus is we want to keep the physical ds constant (corresponding to the production as-built values) we need to change the effective ds in the code.

The appendix (section 6) lists the various parameter sets used in this investigation.

Note that the parameters are only an approximation to the production values. For example we have modeled the penultimate reaction masses as right circular cylinders and in reality they are more complicated. Also no wedges are included for the ERM or CP. But the values are close enough to draw conclusions about the changes which occur when different radii of wire are used at the final stage.

# Observations regarding effect of wire radius

The major effects seen when the wire radius of the final stage in the reaction chain is changed is that:

i) With increasing wire radius, the highest two pitch modes are raised and the pitch isolation is slightly decreased (by less than factor of two at 10 Hz for the wire radii considered). See figure 1.

ii) With increasing wire radius, the highest vertical and roll modes are increased, see figures 2 and 3.

For the ERM with r = 0.2285 mm (default value), highest vertical mode = 18.3 Hz and highest roll mode = 26.1 Hz

For the ERM with r = 0.175 mm (thinner wire), highest vertical mode = 14.4 Hz and highest roll mode = 20.5 Hz

For the thinner CP with r = 0.2285 mm (default value), highest vertical mode = 19.8 Hz and highest roll mode =   28.2 Hz

Other modes and transfer functions are essentially unchanged.

# Conclusions

The isolation required in the reaction chain of the quad is modest compared to the main chain – see for example T060043-00-K. Thus the small decrease in pitch isolation when using thicker wire is not significant. We have no requirement on the frequencies of the highest vertical and roll modes for the reaction chains. These peaks will only weakly couple into the main chain and will have much lower Qs than the main chain which contains silica fibres. We can see no adverse effect in their being at slightly higher frequencies due to the use of thicker wire. The frequencies are similar to those from other wire suspensions (HLTS and HSTS) and are all below 30 Hz.

The advantage of using the same wire, r = 0.2285 mm, is that is simplifies the assembly process for the quadruple suspensions. Having only one set of four wire sizes for the quad for both main and reaction chains makes keeping track of which wire goes where a simpler process. In addition this choice of wire is what has been assumed in the design of parts by the RAL group.  Thus in conclusion we propose to use the same lowest wire radius, namely r = 0.2285 mm (diameter = 0.457 mm) for all quad suspension chains regardless of the mass of the bottom stage.

# Figure 1. Pitch transfer functions for ERM chain (green and blue) and for thin CP chain (turquoise).                            Green : r3 = 0.2285 mm. Blue: r3 = 0.175 mm. Turquoise: r3 = 0.2285 mm.

# Figure 2. Vertical transfer functions for ERM chain (green and blue) and for thin CP chain (turquoise).                            Green : r3 = 0.2285 mm. Blue: r3 = 0.175 mm. Turquoise: r3 = 0.2285 mm.

# Figure 3. Roll transfer functions for ERM chain (green and blue) and for thin CP chain (turquoise).                            Green : r3 = 0.2285 mm. Blue: r3 = 0.175 mm. Turquoise: r3 = 0.2285 mm.

# Appendix

Default parameters in T1000263 (set for noise prototype with silica fibres in final stage)

pend =

stage2: 1

ribbon: 0

g: 9.8100

nx: 0.1300

ny: 0.5000

nz: 0.0840

denn: 4000

mn: 22.1100

Inx: 0.4558

Iny: 0.0712

Inz: 0.4547

ux: 0.1300

uy: 0.5000

uz: 0.0840

den1: 4000

m1: 21.0110

I1x: 0.5174

I1y: 0.0598

I1z: 0.5205

ix: 0.2000

ir: 0.1700

if: 0.0950

den2: 2200

m2: 39.5696

I2x: 0.5666

I2y: 0.4204

I2z: 0.4101

tx: 0.2000

tr: 0.1700

tf: 0.0950

den3: 2200

m3: 39.5696

I3x: 0.5666

I3y: 0.4204

I3z: 0.4101

nwn: 2

nw1: 4

nw2: 4

nw3: 4

bd: 0

rn: 5.2000e-004

r1: 3.5000e-004

r2: 3.1000e-004

r3: 4.0000e-004

r3n: 4.0000e-004

r3m: 2.0000e-004

nl: 0.0150

Yn: 2.1200e+011

Y1: 2.1200e+011

Y2: 2.1200e+011

Y3: 7.2000e+010

twistlength: 0

d3tr: 1.0000e-003

d4tr: 1.0000e-003

sn: 0

su: 0.0030

si: 0.0030

sl: 0.0150

nn0: 0.2500

nn1: 0.0900

n0: 0.2000

n1: 0.0600

n2: 0.1400

n3: 0.1762

n4: 0.1712

n5: 0.1712

ln: 0.4486

l1: 0.3090

l2: 0.3327

l3: 0.6077

dm: -0.0031

dn: 0.0032

d0: -0.0017

d1: 0.0031

d2: 0.0072

d3: -0.0029

d4: -0.0029

mn3: 122.2602

m13: 100.1502

m23: 79.1392

tlnspec: 0.4160

tl1spec: 0.2770

tl2spec: 0.3410

tl3spec: 0.6020

kxn: 100000

kx1: 100000

kx2: 80000

dmspec: 1.0000e-003

dnspec: 1.0000e-003

d0spec: 1.0000e-003

d1spec: 1.0000e-003

d2spec: 0.0100

d3spec: 1.0000e-003

d4spec: 1.0000e-003

kcn: 1.6111e+003

kc1: 1.8114e+003

kc2: 2.6347e+003

kw3n: 4.8255e+006

kw3m: 3.1322e+004

kw3: 3.0921e+004

tln: 0.4160

tl1: 0.2770

tl2: 0.3410

tl3: 0.6020

Mn1: 5.7425e-014

M11: 1.1786e-014

M21: 7.2533e-015

M31: 2.0106e-014

M32: 2.0106e-014

flexn: 0.0041

flex1: 0.0027

flex2: 0.0028

flex3: 0.0039

ufcn: 1.9213

ufc1: 2.0899

ufc2: 1.8366

kwn: 4.0148e+005

kw1: 5.2808e+005

kw2: 3.8475e+005

l_suspoint_to_centreofoptic: 1.6360

l_suspoint_to_bottomofoptic: 1.8060

flex3tr: 0.0039

longpitch1: [0.4332 0.5193 0.9870 1.2331]

longpitch2: [1.5674 2.0072 2.9454 3.4189]

yaw: [0.5981 1.3452 2.4083 3.0462]

transroll1: [0.4625 0.8615 1.0442 2.1240]

transroll2: [2.6925 3.3254 5.0994 12.8743]

vertical: [0.5815 2.3386 3.7604 9.0024]

Note that sum of test mass (m3) and penultimate mass (m2 ) = 2 x 39.57 = 79.14 kg

A.1 End Reaction Mass

ERM mass (m3) = 25.97 kg (340 mm diameter by 130 mm thick silica)

Thus penultimate mass (m2) = 79.14 – 26.0 =  53.14 kg

m2 has been approximated below by a cylinder of same dimensions as the ERM and fictitious density, as given below.

ix: 0.1300

ir: 0.1700

den2: 4502

m2: 53.1369

I2x: 0.7678

I2y: 0.4587

I2z: 0.4587

tx: 0.1300

tr: 0.1700

den3: 2200

m3: 25.9665

I3x: 0.3752

I3y: 0.2242

I3z: 0.2242

Complete baseline parameter set is as below. Note that d3 and d4 are -1.0 mm, which are physical break-off values as noted by Joe Odell in email to NAR 28 May 2010, and taken from D060338-v1. These values are held constant in subsequent analyses.

r3 is set at 0.2285 mm, default value.

pend =

stage2: 1

ribbon: 0

g: 9.8100

nx: 0.1300

ny: 0.5000

nz: 0.0840

denn: 4000

mn: 22.1100

Inx: 0.4558

Iny: 0.0712

Inz: 0.4547

ux: 0.1300

uy: 0.5000

uz: 0.0840

den1: 4000

m1: 21.0110

I1x: 0.5174

I1y: 0.0598

I1z: 0.5205

ix: 0.1300

ir: 0.1700

if: 0.0950

den2: 4502

m2: 53.1369

I2x: 0.7678

I2y: 0.4587

I2z: 0.4587

tx: 0.1300

tr: 0.1700

tf: 0.0950

den3: 2200

m3: 25.9665

I3x: 0.3752

I3y: 0.2242

I3z: 0.2242

nwn: 2

nw1: 4

nw2: 4

nw3: 4

bd: 0

rn: 5.2000e-004

r1: 3.5000e-004

r2: 3.1000e-004

r3: 2.2850e-004

r3n: 2.2850e-004

r3m: 2.2850e-004

nl: 0.0150

Yn: 2.1200e+011

Y1: 2.1200e+011

Y2: 2.1200e+011

Y3: 2.1200e+011

twistlength: 0

d3tr: 1.0000e-003

d4tr: 1.0000e-003

sn: 0

su: 0.0030

si: 0.0030

sl: 0.0150

nn0: 0.2500

nn1: 0.0900

n0: 0.2000

n1: 0.0600

n2: 0.1400

n3: 0.1762

n4: 0.1712

n5: 0.1712

ln: 0.4486

l1: 0.3090

l2: 0.3327

l3: 0.6041

dm: -0.0031

dn: 0.0032

d0: -0.0017

d1: 0.0031

d2: 0.0072

d3: -0.0010

d4: -0.0010

mn3: 122.2245

m13: 100.1145

m23: 79.1035

tlnspec: 0.4160

tl1spec: 0.2770

tl2spec: 0.3410

tl3spec: 0.6020

kxn: 100000

kx1: 100000

kx2: 80000

dmspec: 1.0000e-003

dnspec: 1.0000e-003

d0spec: 1.0000e-003

d1spec: 1.0000e-003

d2spec: 0.0100

d3spec: 0.0016

d4spec: 0.0016

kcn: 1.6111e+003

kc1: 1.8114e+003

kc2: 2.6347e+003

kw3n: 4.6366e+006

kw3m: 1.2115e+005

kw3: 1.1514e+005

tln: 0.4160

tl1: 0.2770

tl2: 0.3410

tl3: 0.6020

Mn1: 5.7425e-014

M11: 1.1786e-014

M21: 7.2533e-015

M31: 2.1411e-015

M32: 2.1411e-015

flexn: 0.0041

flex1: 0.0027

flex2: 0.0028

flex3: 0.0027

ufcn: 1.9213

ufc1: 2.0899

ufc2: 1.5849

kwn: 4.0147e+005

kw1: 5.2808e+005

kw2: 3.8475e+005

l_suspoint_to_centreofoptic: 1.6360

l_suspoint_to_bottomofoptic: 1.8060

flex3tr: 0.0027

longpitch1: [0.4505 0.6128 0.8590 1.3716]

longpitch2: [1.9256 2.8707 3.0690 3.4091]

yaw: [0.6532 1.3450 2.3237 2.9966]

transroll1: [0.4804 0.8508 0.9096 2.0566]

transroll2: [2.7038 3.3251 5.0981 26.0771]

vertical: [0.5825 2.3494 3.7765 18.3095]

Parameter set for thinner wire (as highlighted)

pend =

stage2: 1

ribbon: 0

g: 9.8100

nx: 0.1300

ny: 0.5000

nz: 0.0840

denn: 4000

mn: 22.1100

Inx: 0.4558

Iny: 0.0712

Inz: 0.4547

ux: 0.1300

uy: 0.5000

uz: 0.0840

den1: 4000

m1: 21.0110

I1x: 0.5174

I1y: 0.0598

I1z: 0.5205

ix: 0.1300

ir: 0.1700

if: 0.0950

den2: 4502

m2: 53.1369

I2x: 0.7678

I2y: 0.4587

I2z: 0.4587

tx: 0.1300

tr: 0.1700

tf: 0.0950

den3: 2200

m3: 25.9665

I3x: 0.3752

I3y: 0.2242

I3z: 0.2242

nwn: 2

nw1: 4

nw2: 4

nw3: 4

bd: 0

rn: 5.2000e-004

r1: 3.5000e-004

r2: 3.1000e-004

r3: 1.7500e-004

r3n: 0.0022

r3m: 1.7500e-004

nl: 0.0150

Yn: 2.1200e+011

Y1: 2.1200e+011

Y2: 2.1200e+011

Y3: 2.1200e+011

twistlength: 0

d3tr: 1.0000e-003

d4tr: 1.0000e-003

sn: 0

su: 0.0030

si: 0.0030

sl: 0.0150

nn0: 0.2500

nn1: 0.0900

n0: 0.2000

n1: 0.0600

n2: 0.1400

n3: 0.1762

n4: 0.1712

n5: 0.1712

ln: 0.4486

l1: 0.3090

l2: 0.3327

l3: 0.6040

dm: -0.0031

dn: 0.0032

d0: -0.0017

d1: 0.0031

d2: 0.0072

d3: -0.0010

d4: -0.0010

mn3: 122.2245

m13: 100.1145

m23: 79.1035

tlnspec: 0.4160

tl1spec: 0.2770

tl2spec: 0.3410

tl3spec: 0.6020

kxn: 100000

kx1: 100000

kx2: 80000

dmspec: 1.0000e-003

dnspec: 1.0000e-003

d0spec: 1.0000e-003

d1spec: 1.0000e-003

d2spec: 0.0100

d3spec: 5.6000e-004

d4spec: 5.6000e-004

kcn: 1.6111e+003

kc1: 1.8114e+003

kc2: 2.6347e+003

kw3n: 4.2009e+008

kw3m: 7.1067e+004

kw3: 7.1043e+004

tln: 0.4160

tl1: 0.2770

tl2: 0.3410

tl3: 0.6020

Mn1: 5.7425e-014

M11: 1.1786e-014

M21: 7.2533e-015

M31: 7.3662e-016

M32: 7.3662e-016

flexn: 0.0041

flex1: 0.0027

flex2: 0.0028

flex3: 0.0016

ufcn: 1.9213

ufc1: 2.0899

ufc2: 1.5849

kwn: 4.0147e+005

kw1: 5.2808e+005

kw2: 3.8475e+005

l_suspoint_to_centreofoptic: 1.6360

l_suspoint_to_bottomofoptic: 1.8060

flex3tr: 0.0016

longpitch1: [0.4504 0.5930 0.8582 1.3646]

longpitch2: [1.9244 2.3692 2.9517 3.4088]

yaw: [0.6531 1.3431 2.3233 2.9965]

transroll1: [0.4803 0.8448 0.9095 2.0566]

transroll2: [2.7021 3.3241 5.0981 20.5003]

vertical: [0.5825 2.3485 3.7753 14.3941]

A.2 Compensator plate (thin version)

CP mass (m3 below) = 20.0 kg (340 mm diameter by 100 mm thick silica)

Thus penultimate mass (m2 below) = 79.14 – 20.0 =  59.14 kg

m2 has been approximated below by a cylinder of same dimensions as the ERM (and the original CP) and fictitious density, as given below.

ix: 0.1300

ir: 0.1700

den2: 5011

m2: 59.1447

I2x: 0.8546

I2y: 0.5106

I2z: 0.5106

tx: 0.1000

tr: 0.1700

den3: 2200

m3: 19.9742

I3x: 0.2886

I3y: 0.1610

I3z: 0.1610

Complete parameter set is as below. Note that d3 and d4 are -1.0 mm, which are physical break-off values as noted by Joe Odell in email to NAR 28 May 2010, and taken from D060338-v1. These values are held constant.

r3 is set at 0.2285 mm, default value.

pend =

stage2: 1

ribbon: 0

g: 9.8100

nx: 0.1300

ny: 0.5000

nz: 0.0840

denn: 4000

mn: 22.1100

Inx: 0.4558

Iny: 0.0712

Inz: 0.4547

ux: 0.1300

uy: 0.5000

uz: 0.0840

den1: 4000

m1: 21.0110

I1x: 0.5174

I1y: 0.0598

I1z: 0.5205

ix: 0.1300

ir: 0.1700

if: 0.0950

den2: 5011

m2: 59.1447

I2x: 0.8546

I2y: 0.5106

I2z: 0.5106

tx: 0.1000

tr: 0.1700

tf: 0.0950

den3: 2200

m3: 19.9742

I3x: 0.2886

I3y: 0.1610

I3z: 0.1610

nwn: 2

nw1: 4

nw2: 4

nw3: 4

bd: 0

rn: 5.2000e-004

r1: 3.5000e-004

r2: 3.1000e-004

r3: 2.2850e-004

r3n: 2.2850e-004

r3m: 2.2850e-004

nl: 0.0150

Yn: 2.1200e+011

Y1: 2.1200e+011

Y2: 2.1200e+011

Y3: 2.1200e+011

twistlength: 0

d3tr: 1.0000e-003

d4tr: 1.0000e-003

sn: 0

su: 0.0030

si: 0.0030

sl: 0.0150

nn0: 0.2500

nn1: 0.0900

n0: 0.2000

n1: 0.0600

n2: 0.1400

n3: 0.1762

n4: 0.1712

n5: 0.1712

ln: 0.4486

l1: 0.3090

l2: 0.3327

l3: 0.6041

dm: -0.0031

dn: 0.0032

d0: -0.0017

d1: 0.0031

d2: 0.0072

d3: -0.0010

d4: -0.0010

mn3: 122.2399

m13: 100.1299

m23: 79.1189

tlnspec: 0.4160

tl1spec: 0.2770

tl2spec: 0.3410

tl3spec: 0.6020

kxn: 100000

kx1: 100000

kx2: 80000

dmspec: 1.0000e-003

dnspec: 1.0000e-003

d0spec: 1.0000e-003

d1spec: 1.0000e-003

d2spec: 0.0100

d3spec: 0.0020

d4spec: 0.0020

kcn: 1.6111e+003

kc1: 1.8114e+003

kc2: 2.6347e+003

kw3n: 4.6366e+006

kw3m: 1.2115e+005

kw3: 1.1513e+005

tln: 0.4160

tl1: 0.2770

tl2: 0.3410

tl3: 0.6020

Mn1: 5.7425e-014

M11: 1.1786e-014

M21: 7.2533e-015

M31: 2.1411e-015

M32: 2.1411e-015

flexn: 0.0041

flex1: 0.0027

flex2: 0.0028

flex3: 0.0030

ufcn: 1.9213

ufc1: 2.0899

ufc2: 1.5023

kwn: 4.0147e+005

kw1: 5.2808e+005

kw2: 3.8475e+005

l_suspoint_to_centreofoptic: 1.6360

l_suspoint_to_bottomofoptic: 1.8060

flex3tr: 0.0030

longpitch1: [0.4589 0.6188 0.8090 1.3760]

longpitch2: [1.9046 2.9211 3.3219 3.4078]

yaw: [0.6655 1.3437 2.2629 2.9686]

transroll1: [0.4906 0.8249 0.8796 2.0383]

transroll2: [2.7045 3.3232 5.0981 28.1695]

vertical: [0.5825 2.3499 3.7772 19.7787]