| Originator | Cognizant Engineer | Ext./Phone# | Project | Account Number |
| --- | --- | --- | --- | --- |
| Name Brian O’Reilly | Name Ken Mason | # 225-686-3116 | LIGO |  |

| Dwg/Part Number | Rev | Part Description | Serial Number | Qty |
| --- | --- | --- | --- | --- |
|  |  | 3/8-16x2-1/2” Silver plated SHCS 1/2-13x2” Silver plated SHCS |  | 50 50 |

| Used In (next higher assembly): | HAM-ISI |
| --- | --- |

| Vendor Name | PO/Contract Number |
| --- | --- |

Data Package, Receiving/Inspection Remarks:

| Inspection Required Y/N | Visual Damage Y/N | Comments | Name/ Initials | Date Comp. |
| --- | --- | --- | --- | --- |
| N |  |  |  |  |

Process Flow:

| # | Operation | Start Date | Work Area | Instructions | Name/ Initials | Date Comp. |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Clean |  | LLO | Per E960022 | T.E. | 1/16/08 |
| 2 | Vacuum Bake |  | LLO | per E960022 to a temperature of 200C N.B.: Take outgassing measurements during cool down at approximately room temperature. | T.E. | 1/21/08 |
| 3 | Control Point |  | NA | Review/approve RGA scan # | BO’R | 1/22/08 |
| 4 | Wrap & Tag vacuum clean parts per E960022-A |  | LLO | Wrap (UHV foil) and bag (CP Stat or equiv.) per E960022. | T.E. | 1/22/08 |
| 5 | Deliver/File paperwork |  | LLO | Make 2 copies of the Traveler. File one copy with the DCC. Ship parts to location for final assembly | BO’R | 1/25/08 |
| END: Go to Traveler or procedure associated with next higher assembly processing |  |  |  |  |  |  |

Special Instructions (Handling/Packaging Constraints, Remarks, etc.) or Notes:

These bolts were silver-plated through Lavallee Machine in MA (a plater they know). The are used for the spring posts on the HAM-ISI assembly.