LIGO Laboratory / LIGO Scientific Collaboration

LIGO-T1100472-v37	Advanced LIGO	January 24, 2025

ADC and DAC Channel Usage for ISC

Rich Abbott, Peter Fritschel, Daniel Sigg

Distribution of this document:

LIGO Scientific Collaboration

This is an internal working note

of the LIGO Laboratory.

| California Institute of Technology LIGO Project – MS 18-34 1200 E. California Blvd. Pasadena, CA 91125 Phone (626) 395-2129 Fax (626) 304-9834 E-mail: info@ligo.caltech.edu | Massachusetts Institute of Technology LIGO Project – NW22-295 185 Albany St Cambridge, MA 02139 Phone (617) 253-4824 Fax (617) 253-7014 E-mail: info@ligo.mit.edu |
| --- | --- |
| LIGO Hanford Observatory P.O. Box 159 Richland WA 99352 Phone 509-372-8106 Fax 509-372-8137 | LIGO Livingston Observatory P.O. Box 940 Livingston, LA 70754 Phone 225-686-3100 Fax 225-686-7189 |

http://www.ligo.caltech.edu/

# Purpose

This document lists the specific ADC and DAC channels used within the ISC I/O expansion chassis. In the following tables, the entries given in the ‘Signal’ column are not meant to be the exact DAQ channel name for that signal (though they may be); rather the entries are intended as descriptors to identify the actual hardware channel that is connected to a given ADC/DAC channel.

The reference document for the actual DAQ channel names is T1000264, List of ISC Photodetectors in Advanced LIGO.

# ASC-IO I/O Chassis

| Card | AA/AI conn. | ADC/DAC Chns. | Signal |  |  |
| --- | --- | --- | --- | --- | --- |
| ADC 0 | DB9_1 | 1 | IMC-WFS_A_RF | Seg 1 | Q-phase |
|  |  | 2 |  |  | I-phase |
|  |  | 3 |  | Seg 2 | Q-phase |
|  |  | 4 |  |  | I-phase |
|  | DB9_2 | 5 |  | Seg 3 | Q-phase |
|  |  | 6 |  |  | I-phase |
|  |  | 7 |  | Seg 4 | Q-phase |
|  |  | 8 |  |  | I-phase |
|  | DB9_3 | 9 | IMC-WFS_B_RF | Seg 1 | Q-phase |
|  |  | 10 |  |  | I-phase |
|  |  | 11 |  | Seg 2 | Q-phase |
|  |  | 12 |  |  | I-phase |
|  | DB9_4 | 13 |  | Seg 3 | Q-phase |
|  |  | 14 |  |  | I-phase |
|  |  | 15 |  | Seg 4 | Q-phase |
|  |  | 16 |  |  | I-phase |
|  | DB9_5 | 17-20 | IMC-WFS_A_DC | Segs 1-4 | DC Outputs of WFS |
|  | DB9_6 | 21-24 | IMC-WFS_B_DC | Segs 1-4 |  |
|  | DB9_7 | 25 | ALS-C_SHG_IR_LF |  | Aux. signals concentrator 3 REFL PD Amp 1-4 |
|  |  | 26 | ALS-C_SHG_GR_LF |  |  |
|  |  | 27 | ALS-C_TRX_A_LF |  |  |
|  |  | 28 | ALS-C_TRY_A_LF |  |  |
|  | DB9_8 | 29-30 | Unused |  |  |
|  |  | 31 | Unused |  |  |
|  |  | 32 | Duotone |  |  |

| Card | AA/AI conn. | ADC/DAC Chs. | Signal |  |  |
| --- | --- | --- | --- | --- | --- |
| ADC 1 | DB9_1 | 1-4 | IO QPD: SM2 Transmission |  |  |
|  | DB9_2 | 5-8 | IO QPD: MC2 Transmission |  |  |
|  | DB9_3 | 9 | PD 1: post-EOM DC |  | 4 ch Generic PD interface: PSL/IO table |
|  |  | 10 | PD 2: post-power control DC |  |  |
|  |  | 11 | PD 1: post-EOM AC |  |  |
|  |  | 12 | PD 2: post-power control AC |  |  |
|  | DB9_4 | 13 | PD 1: unused |  | 4 ch Generic PD interface: IOT1 |
|  |  | 14 | PD 2: IMC-REFL_TRIG_LF |  |  |
|  |  | 15-16 | PD 3,4: unused |  |  |
|  | DB9_5 | 17 | ASC-AS_A_RF42 | Seg 1 | Q-phase |
|  |  | 18 |  |  | I-phase |
|  |  | 19 |  | Seg 2 | Q-phase |
|  |  | 20 |  |  | I-phase |
|  | DB9_6 | 21 |  | Seg 3 | Q-phase |
|  |  | 22 |  |  | I-phase |
|  |  | 23 |  | Seg 4 | Q-phase |
|  |  | 24 |  |  | I-phase |
|  | DB9_7 | 25 | ASC-AS_B_RF42 | Seg 1 | Q-phase |
|  |  | 26 |  |  | I-phase |
|  |  | 27 |  | Seg 2 | Q-phase |
|  |  | 28 |  |  | I-phase |
|  | DB9_8 | 29 |  | Seg 3 | Q-phase |
|  |  | 30 |  |  | I-phase |
|  |  | 31 |  | Seg 4 | Q-phase |
|  |  | 32 |  |  | I-phase |

| Card | AA/AI conn. | ADC/DAC chan. | Signal |  |  |
| --- | --- | --- | --- | --- | --- |
| ADC 2 | DB9_1 | 1 | ASC-REFL_A_RF9 | Seg 1 | Q-phase |
|  |  | 2 |  |  | I-phase |
|  |  | 3 |  | Seg 2 | Q-phase |
|  |  | 4 |  |  | I-phase |
|  | DB9_2 | 5 |  | Seg 3 | Q-phase |
|  |  | 6 |  |  | I-phase |
|  |  | 7 |  | Seg 4 | Q-phase |
|  |  | 8 |  |  | I-phase |
|  | DB9_3 | 9 | ASC-REFL_A_RF45 | Seg 1 | Q-phase |
|  |  | 10 |  |  | I-phase |
|  |  | 11 |  | Seg 2 | Q-phase |
|  |  | 12 |  |  | I-phase |
|  | DB9_4 | 13 |  | Seg 3 | Q-phase |
|  |  | 14 |  |  | I-phase |
|  |  | 15 |  | Seg 4 | Q-phase |
|  |  | 16 |  |  | I-phase |
|  | DB9_5 | 17 | ASC-REFL_B_RF9 | Seg 1 | Q-phase |
|  |  | 18 |  |  | I-phase |
|  |  | 19 |  | Seg 2 | Q-phase |
|  |  | 20 |  |  | I-phase |
|  | DB9_6 | 21 |  | Seg 3 | Q-phase |
|  |  | 22 |  |  | I-phase |
|  |  | 23 |  | Seg 4 | Q-phase |
|  |  | 24 |  |  | I-phase |
|  | DB9_7 | 25 | ASC-REFL_B_RF45 | Seg 1 | Q-phase |
|  |  | 26 |  |  | I-phase |
|  |  | 27 |  | Seg 2 | Q-phase |
|  |  | 28 |  |  | I-phase |
|  | DB9_8 | 29 |  | Seg 3 | Q-phase |
|  |  | 30 |  |  | I-phase |
|  |  | 31 |  | Seg 4 | Q-phase |
|  |  | 32 |  |  | I-phase |
| Card | AA/AI conn. | ADC/DAC chan. | Signal |  |  |
| ADC 3 | DB9_1 | 1 | ASC-POP_X_RF | Seg 1 | Q-phase |
|  |  | 2 |  |  | I-phase |
|  |  | 3 |  | Seg 2 | Q-phase |
|  |  | 4 |  |  | I-phase |
|  | DB9_2 | 5 |  | Seg 3 | Q-phase |
|  |  | 6 |  |  | I-phase |
|  |  | 7 |  | Seg 4 | Q-phase |
|  |  | 8 |  |  | I-phase |
|  | DB9_3 | 9-12 | JAC-WFS_A_DC |  | DC Outputs of WFS |
|  | DB9_4 | 13-16 | JAC-WFS_B_DC |  |  |
|  | DB9_5 | 17 | ASC-AS_A_RF72 | Seg 1 | Q-phase |
|  |  | 18 |  |  | I-phase |
|  |  | 19 |  | Seg 2 | Q-phase |
|  |  | 20 |  |  | I-phase |
|  | DB9_6 | 21 |  | Seg 3 | Q-phase |
|  |  | 22 |  |  | I-phase |
|  |  | 23 |  | Seg 4 | Q-phase |
|  |  | 24 |  |  | I-phase |
|  | DB9_7 | 25 | ASC-AS_B_RF72 | Seg 1 | Q-phase |
|  |  | 26 |  |  | I-phase |
|  |  | 27 |  | Seg 2 | Q-phase |
|  |  | 28 |  |  | I-phase |
|  | DB9_8 | 29 |  | Seg 3 | Q-phase |
|  |  | 30 |  |  | I-phase |
|  |  | 31 |  | Seg 4 | Q-phase |
|  |  | 32 |  |  | I-phase |

| Card | AA/AI conn. |  | ADC/DAC chan. |  | Signal |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADC 4 | DB9_1 |  | 1 |  | ASC-AS_A_RF45 |  | Seg 1 | Q-phase |
|  |  |  | 2 |  |  |  |  | I-phase |
|  |  |  | 3 |  |  |  | Seg 2 | Q-phase |
|  |  |  | 4 |  |  |  |  | I-phase |
|  | DB9_2 |  | 5 |  |  |  | Seg 3 | Q-phase |
|  |  |  | 6 |  |  |  |  | I-phase |
|  |  |  | 7 |  |  |  | Seg 4 | Q-phase |
|  |  |  | 8 |  |  |  |  | I-phase |
|  | DB9_3 |  | 9 |  | ASC-AS_A_RF36 |  | Seg 1 | Q-phase |
|  |  |  | 10 |  |  |  |  | I-phase |
|  |  |  | 11 |  |  |  | Seg 2 | Q-phase |
|  |  |  | 12 |  |  |  |  | I-phase |
|  | DB9_4 |  | 13 |  |  |  | Seg 3 | Q-phase |
|  |  |  | 14 |  |  |  |  | I-phase |
|  |  |  | 15 |  |  |  | Seg 4 | Q-phase |
|  |  |  | 16 |  |  |  |  | I-phase |
|  | DB9_5 |  | 17 |  | ASC-AS_B_RF45 |  | Seg 1 | Q-phase |
|  |  |  | 18 |  |  |  |  | I-phase |
|  |  |  | 19 |  |  |  | Seg 2 | Q-phase |
|  |  |  | 20 |  |  |  |  | I-phase |
|  | DB9_6 |  | 21 |  |  |  | Seg 3 | Q-phase |
|  |  |  | 22 |  |  |  |  | I-phase |
|  |  |  | 23 |  |  |  | Seg 4 | Q-phase |
|  |  |  | 24 |  |  |  |  | I-phase |
|  | DB9_7 |  | 25 |  | ASC-AS_B_RF36 |  | Seg 1 | Q-phase |
|  |  |  | 26 |  |  |  |  | I-phase |
|  |  |  | 27 |  |  |  | Seg 2 | Q-phase |
|  |  |  | 28 |  |  |  |  | I-phase |
|  | DB9_8 |  | 29 |  |  |  | Seg 3 | Q-phase |
|  |  |  | 30 |  |  |  |  | I-phase |
|  |  |  | 31 |  |  |  | Seg 4 | Q-phase |
|  |  |  | 32 |  |  |  |  | I-phase |
| Card |  | AA/AI conn. |  | ADC/DAC Chs. |  | Signal |  |  |  |
| ADC 5 |  | DB9_1 |  | 1-4 |  | ASC-REFL_A_DC |  | DC Outputs of WFS |  |
|  |  | DB9_2 |  | 5-8 |  | ASC-REFL_B_DC |  |  |  |
|  |  | DB9_3 |  | 9-12 |  | ASC-AS_A_DC |  |  |  |
|  |  | DB9_4 |  | 13-16 |  | ASC-AS_B_DC |  |  |  |
|  |  | DB9_5 |  | 17-20 |  | LO_A_DC |  |  |  |
|  |  | DB9_6 |  | 21-24 |  | LO_B_DC |  |  |  |
|  |  | DB9_7 |  | 25-28 |  | PSL-BES_A_DC |  |  |  |
|  |  | DB9_8 |  | 29-32 |  | ASC-POP_X_DC |  |  |  |

| Card | AA/AI conn. | ADC/DAC Chs. | Signal |
| --- | --- | --- | --- |
| ADC 6 | DB9_1 | 1-4 | ASC-POP_A (QPD) |
|  | DB9_2 | 5-8 | ASC-POP_B (QPD) |
|  | DB9_3 | 9-12 | ASC-AS_C (QPD) |
|  | DB9_4 | 13-16 | ASC-OMC_A (QPD) |
|  | DB9_5 | 17-20 | ASC-OMC_B (QPD) |
|  | DB9_6 | 21-24 | ASC-OMC_C (QPD) |
|  | DB9_7 | 25-28 | ASC-OMC_D (QPD) |
|  | DB9_8 | 29-32 | ASC-LO_C (QPD) |

| Card |  | AA/AI conn. |  | ADC/DAC Chns. |  | Signal |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADC 7 |  | DB9_1 |  | 1 |  | LO_A_RF |  | Seg 1 |  | Q-phase |  |
|  |  |  |  | 2 |  |  |  |  |  | I-phase |  |
|  |  |  |  | 3 |  |  |  | Seg 2 |  | Q-phase |  |
|  |  |  |  | 4 |  |  |  |  |  | I-phase |  |
|  |  | DB9_2 |  | 5 |  |  |  | Seg 3 |  | Q-phase |  |
|  |  |  |  | 6 |  |  |  |  |  | I-phase |  |
|  |  |  |  | 7 |  |  |  | Seg 4 |  | Q-phase |  |
|  |  |  |  | 8 |  |  |  |  |  | I-phase |  |
|  |  | DB9_3 |  | 9 |  | LO_B_RF |  | Seg 1 |  | Q-phase |  |
|  |  |  |  | 10 |  |  |  |  |  | I-phase |  |
|  |  |  |  | 11 |  |  |  | Seg 2 |  | Q-phase |  |
|  |  |  |  | 12 |  |  |  |  |  | I-phase |  |
|  |  | DB9_4 |  | 13 |  |  |  | Seg 3 |  | Q-phase |  |
|  |  |  |  | 14 |  |  |  |  |  | I-phase |  |
|  |  |  |  | 15 |  |  |  | Seg 4 |  | Q-phase |  |
|  |  |  |  | 16 |  |  |  |  |  | I-phase |  |
| ADC 7 |  | DB9_5 |  | 1 |  | JAC-WFS_A_RF |  | Seg 1 |  | Q-phase |  |
|  |  |  |  | 2 |  |  |  |  |  | I-phase |  |
|  |  |  |  | 3 |  |  |  | Seg 2 |  | Q-phase |  |
|  |  |  |  | 4 |  |  |  |  |  | I-phase |  |
|  |  | DB9_6 |  | 5 |  |  |  | Seg 3 |  | Q-phase |  |
|  |  |  |  | 6 |  |  |  |  |  | I-phase |  |
|  |  |  |  | 7 |  |  |  | Seg 4 |  | Q-phase |  |
|  |  |  |  | 8 |  |  |  |  |  | I-phase |  |
|  |  | DB9_7 |  | 9 |  | JAC-WFS_B_RF |  | Seg 1 |  | Q-phase |  |
|  |  |  |  | 10 |  |  |  |  |  | I-phase |  |
|  |  |  |  | 11 |  |  |  | Seg 2 |  | Q-phase |  |
|  |  |  |  | 12 |  |  |  |  |  | I-phase |  |
|  |  | DB9_8 |  | 13 |  |  |  | Seg 3 |  | Q-phase |  |
|  |  |  |  | 14 |  |  |  |  |  | I-phase |  |
|  |  |  |  | 15 |  |  |  | Seg 4 |  | Q-phase |  |
|  |  |  |  | 16 |  |  |  |  |  | I-phase |  |

| Card | AA/AI conn. | ADC/DAC Chs. | Signal |
| --- | --- | --- | --- |
| DAC 0 | DB9_1 | 1 | SQZ-OPO_SERVO_EXC |
|  |  | 2 | SQZ-SHG_SERVO_EXC |
|  |  | 3 | SQZ-LO_SERVO_EXC |
|  |  | 4 | SQZ-CLF_SERVO_EXC |
|  | DB9_2 | 5-6 | Unused |
|  |  | 7-8 | Unused |
|  | DB9_3 | 9 | IO Input beam tip-tilt PZT drive: pitch |
|  |  | 10 | IO Input beam tip-tilt PZT drive: yaw |
|  |  | 11 | ASC-POP_X_PIT |
|  |  | 12 | ASC-POP_X_YAW |
|  | DB9_4 | 13-15 | Unused |
|  |  | 16 | DuoTone |

# LSC I/O Chassis

| Card | AA/AI conn. | ADC/DAC Chns. | Signal |  |
| --- | --- | --- | --- | --- |
| ADC 0 | DB9_1 | 1 | PD 1: ALS-C_DIFF_A_LF | 4 ch Generic PD interface: ISCT1 |
|  |  | 2 | PD 2: ALS-C_COMM_A_LF |  |
|  |  | 3 | PD 3: LSC-REFLAIR_B_LF |  |
|  |  | 4 | PD 4: LSC-POPAIR_B_LF |  |
|  | DB9_2 | 5 | LSC-POPAIR_A | DC Outputs w/ whitening |
|  |  | 6 | LSC-REFLAIR_A |  |
|  |  | 7 | LSC-POP_A |  |
|  |  | 8 | LSC-REFL_A |  |
|  | DB9_3 | 9 | OMC-BIAS_DCPD_A |  |
|  |  | 10 | OMC-BIAS_DCPD_B |  |
|  |  | 11 | OMC-BIAS_DCPD_C |  |
|  |  | 12 | OMC-BIAS_DCPD_D |  |
|  | DB9_4 | 13 | OMC B PZT1 Monitor: LV DC path |  |
|  |  | 14 | OMC B PZT1 Monitor: LV AC path |  |
|  |  | 15 | OMC B PZT2 Monitor: HV DC path |  |
|  |  | 16 | OMC B PZT2 Monitor: HV AC path |  |
|  | DB9_5 | 17 | OMC A PZT1 Monitor: LV DC path |  |
|  |  | 18 | OMC A PZT1 Monitor: LV AC path |  |
|  |  | 19 | OMC A PZT2 Monitor: HV DC path |  |
|  |  | 20 | OMC A PZT2 Monitor: HV AC path |  |
|  | DB9_6 | 21 | LSC-REFL_SERVO_ERR, CM Servo, I monitor |  |
|  |  | 22 | LSC-REFL_SERVO_CTRL, CM Servo, Fast monitor |  |
|  |  | 23 | LSC-REFL_SERVO_SLOW, CM Servo, Slow monitor |  |
|  |  | 24 | OMC-REFL_A_LF |  |
|  | DB9_7 | 25 | IMC-I, IMC Servo, I monitor |  |
|  |  | 26 | IMC-F, IMC Servo, Fast monitor |  |
|  |  | 27 | IMC-L, IMC Servo, Slow monitor |  |
|  |  | 28 | Spare LF |  |
|  | DB9_8 | 29 | OMC-REFL_A | Q-phase |
|  |  | 30 |  | I-phase |
|  |  | 31 | Unused or Duotone (DAC) |  |
|  |  | 32 | Duotone |  |

| Card | AA/AI conn. | ADC/DAC Chns. | Signal |  |  |
| --- | --- | --- | --- | --- | --- |
| ADC 1 | DB9_1 | 1 | LSC-POPAIR_B | RF18 | Q-phase |
|  |  | 2 |  |  | I-phase |
|  |  | 3 |  | RF90 | Q-phase |
|  |  | 4 |  |  | I-phase |
|  | DB9_2 | 5 | LSC-REFLAIR_B | RF27 | Q-phase |
|  |  | 6 |  |  | I-phase |
|  |  | 7 |  | RF135 | Q-phase |
|  |  | 8 |  |  | I-phase |
|  | DB9_3 | 9 | SQZ-OMC_TRANS | RF3 | Q-phase |
|  |  | 10 |  |  | I-phase |
|  |  | 11 | SQZ-HD_DIFF | RF3 | Q-phase |
|  |  | 12 |  |  | I-phase |
|  | DB9_4 | 13 | SQZ-OPO_REFL | RF80 | Q-phase |
|  |  | 14 |  |  | I-phase |
|  |  | 15 | SQZ-SHG_TRANS | RF24 | Q-phase |
|  |  | 16 |  |  | I-phase |
|  | DB9_5 | 17 | LSC-POPAIR_A | RF9 | Q-phase |
|  |  | 18 |  |  | I-phase |
|  |  | 19 |  | RF45 | Q-phase |
|  |  | 20 |  |  | I-phase |
|  | DB9_6 | 21 | LSC-REFLAIR_A | RF9 | Q-phase |
|  |  | 22 |  |  | I-phase |
|  |  | 23 |  | RF45 | Q-phase |
|  |  | 24 |  |  | I-phase |
|  | DB9_7 | 25 | LSC-POP_A | RF9 | Q-phase |
|  |  | 26 |  |  | I-phase |
|  |  | 27 |  | RF45 | Q-phase |
|  |  | 28 |  |  | I-phase |
|  | DB9_8 | 29 | LSC-REFL_A | RF9 | Q-phase |
|  |  | 30 |  |  | I-phase |
|  |  | 31 |  | RF45 | Q-phase |
|  |  | 32 |  |  | I-phase |

| Card | AA/AI conn. | ADC/DAC Chns. | Signal |  |  |
| --- | --- | --- | --- | --- | --- |
| ADC 2 | DB9_1 | 1 | IMC-REFL_A_DC |  | DC Outputs w/ whitening |
|  |  | 2 | LSC-REFL_B_LF |  |  |
|  |  | 3 | JAC-REFL_A_DC |  |  |
|  |  | 4 | PSL TTFSS PD (power only) |  |  |
|  | DB9_2 | 5 | ALS-C_COMM_A_RF_ERR (PFD signal ALS Comm) |  |  |
|  |  | 6 | LSC-REFL_B_RF9_ERR (Demod signal REFL) |  |  |
|  |  | 7 | ALS-C_REFL_DC_ERR (DC signal from REFL_A PD) |  |  |
|  |  | 8 | LSC-REFL_A_RF9_ERR (Demod signal REFL) |  |  |
|  | DB9_3 | 9 | LSC EXTRA_AI_1 |  |  |
|  |  | 10 | LSC EXTRA_AI_2 |  |  |
|  |  | 11 | LSC-REFL_B | RF9 | Q-phase |
|  |  | 12 |  |  | I-phase |
|  | DB9_4 | 13 | ALS-C_COMM_PLL_ERR |  |  |
|  |  | 14 | ALS-C_COMM_PLL_CTRL |  |  |
|  |  | 15 | ALS-C_DIFF_PLL_ERR |  |  |
|  |  | 16 | ALS-C_DIFF_PLL_CTRL |  |  |
|  | DB9_5 | 17 | LSC-MOD_RF9_AM_ERR |  |  |
|  |  | 18 | LSC-MOD_RF9_AM_CTRL |  |  |
|  |  | 19 | LSC-MOD_RF9_AM_AC |  |  |
|  |  | 20 | LSC-MOD_RF9_AM_DC |  |  |
|  | DB9_6 | 21 | LSC-MOD_RF45_AM_ERR |  |  |
|  |  | 22 | LSC-MOD_RF45_AM_CTRL |  |  |
|  |  | 23 | LSC-MOD_RF45_AM_AC |  |  |
|  |  | 24 | LSC-MOD_RF45_AM_DC |  |  |
|  | DB9_7 | 25 | SQZ-OPO_ISS_ERR |  |  |
|  |  | 26 | SQZ-OPO_ISS_CTRL |  |  |
|  |  | 27 | SQZ-CLF_ISS_ERR |  |  |
|  |  | 28 | SQZ-CLF_ISS_CTRL |  |  |
|  | DB9_8 | 29 | IMC_REFL_A | RF24 | Q-phase |
|  |  | 30 |  |  | I-phase |
|  |  | 31 | JAC-REFL_A | RF42 | Q-phase |
|  |  | 32 |  |  | I-phase |

| Card | AA/AI conn. | ADC/DAC Chns. | Signal |  |
| --- | --- | --- | --- | --- |
| ADC 3 | DB9_1 | 1 | SQZ-OPO_SERVO_ERR, CM Servo, I monitor |  |
|  |  | 2 | SQZ-OPO_SERVO_CTRL, CM Servo, Fast monitor |  |
|  |  | 3 | SQZ-OPO_SERVO_SLOW, CM Servo, Slow monitor |  |
|  |  | 4 | SQZ-CLF_SERVO_ERR, CM Servo, I monitor |  |
|  | DB9_2 | 5 | SQZ-SHG_SERVO_ERR, CM Servo, I monitor |  |
|  |  | 6 | SQZ-SHG_SERVO_CTRL, CM Servo, Fast monitor |  |
|  |  | 7 | SQZ-SHG_SERVO_SLOW, CM Servo, Slow monitor |  |
|  |  | 8 | SQZ-CLF_SERVO_CTRL, CM Servo, Fast monitor |  |
|  | DB9_3 | 9 | SQZ-LO_SERVO_ERR, CM Servo, I monitor |  |
|  |  | 10 | SQZ-LO_SERVO_CTRL, CM Servo, Fast monitor |  |
|  |  | 11 | SQZ-LO_SERVO_SLOW, CM Servo, Slow monitor |  |
|  |  | 12 | SQZ-CLF_SERVO_SLOW, CM Servo, Slow monitor |  |
|  | DB9_4 | 13 | SQZ-HD_B_DC |  |
|  |  | 14 | SQZ-HD_DIFF_DC |  |
|  |  | 15 | SQZ-HD_A_DC |  |
|  |  | 16 | Unused |  |
|  | DB9_5 | 17 | SQZ-FIBR_MIXER |  |
|  |  | 18 | SQZ-FIBR_PZT |  |
|  |  | 19 | SQZ-FIBR_EOMRMS |  |
|  |  | 20 | SQZ-FIBR_SLOW |  |
|  | DB9_6 | 21 | PD1: SQZ-PMC_TRANS_LF | 4 ch Generic PD interface: SQZT0 w/ whitening |
|  |  | 22 | PD2: SQZ-SHG_TRANS_LF |  |
|  |  | 23 | PD3: SQZ-LASER_IR_LF |  |
|  |  | 24 | PD4: SQZ-SHG_GR_LF |  |
|  | DB9_7 | 25 | SQZ-OPO_TRANS_LF | DC Outputs w/ whitening |
|  |  | 26 | SQZ-OPO_REFL_REJECTED_LF |  |
|  |  | 27 | SQZ- FIBR_TRANS _LF |  |
|  |  | 28 | SQZ-FC_REFL_REJECTED _LF |  |
|  | DB9_8 | 29 | SQZ-EXTRA_AI_1 |  |
|  |  | 30 | SQZ-EXTRA_AI_2 |  |
|  |  | 31 | SQZ-EXTRA_AI_3 |  |
|  |  | 32 | SQZ-EXTRA_AI_4 |  |

| Card | AA/AI conn. | ADC/DAC Chns. | Signal |  |  |
| --- | --- | --- | --- | --- | --- |
| ADC 4 | DB9_1 | 1 | SQZ-FC_SERVO_ERR, CM Servo, I monitor |  |  |
|  |  | 2 | SQZ-FC_SERVO_CTRL, CM Servo, Fast monitor |  |  |
|  |  | 3 | SQZ-FC_SERVO_SLOW, CM Servo, Slow monitor |  |  |
|  |  | 4 | Unused |  |  |
|  | DB9_3 | 5 | SQZ-CLF_REFL | RF6 | Q-phase |
|  |  | 6 |  |  | I-phase |
|  |  | 7 | SQZ-FC_REFL | RF40 | Q-phase |
|  |  | 8 |  |  | I-phase |
|  | DB9_3 | 9 | Unused |  |  |
|  |  | 10 | SQZ-PMC_REFL_LF |  |  |
|  |  | 11 | SQZ-FIBR_PD_LF |  |  |
|  |  | 12 | SQZ-ADF_VCXO_RF203_I |  |  |
|  | DB9_4 | 13 | SQZ-PMC_REFL | RF35 | Q-phase |
|  |  | 14 |  |  | I-phase |
|  |  | 15 | Spare | RF | Q-phase |
|  |  | 16 |  |  | I-phase |
|  | DB9_5 | 17 | SQZ-PMC_SERVO_ERR, CM Servo, I monitor |  |  |
|  |  | 18 | SQZ-PMC_SERVO_CTRL, CM Servo, Fast monitor |  |  |
|  |  | 19 | SQZ-PMC_SERVO_SLOW, CM Servo, Slow monitor |  |  |
|  |  | 20 | Unused |  |  |
|  | DB9_6 | 21 | MOTION_C_SHUTTER_H_TRIGGER (CLF path trigger PD) |  | 4 ch Generic PD interface: SQZT7 |
|  |  | 22 | SQZ-OPO_IR |  |  |
|  |  | 23 | SQZ-CLF_DC |  |  |
|  |  | 24 | SQZ-OPO_DC |  |  |
|  | DB9_7 | 25 | SQZ-CLF_REFL_LF |  | DC Outputs w/ whitening |
|  |  | 26 | SQZ-OPO_REFL_LF |  |  |
|  |  | 27 | SQZ-FC_REFL_LF |  |  |
|  |  | 28 | SQZ-FC_BEAT_LF |  |  |
|  | DB9_8 | 29 | OMC A PZT3 Monitor: HV DC path |  |  |
|  |  | 30 | OMC A PZT3 Monitor: HV AC path |  |  |
|  |  | 31 | OMC B PZT3 Monitor: HV DC path |  |  |
|  |  | 32 | OMC B PZT3 Monitor: HV AC path |  |  |
| Card | AA/AI conn. | ADC/DAC Chns. | Signal |  |  |
| ADC 5 526kHz | DB37 | 1 | SQZ-OMC_TRANS | RF3 | I-phase |
|  |  | 2 |  |  | Q-phase |
|  |  | 3 | SQZ-HD_DIFF | RF3 | I-phase |
|  |  | 4 |  |  | Q-phase |
|  |  | 5 | Unused |  |  |
|  |  | 6 | Unused |  |  |
|  |  | 7 | SQZ-FC_BEAT_MF |  |  |
|  |  | 8 | SQZ-RLF_VCXO_RF200_I_MF |  |  |
|  |  | 9 | SQZ-FC_QPD_A | Seg 3 | MF |
|  |  | 10 |  | Seg 4 | MF |
|  |  | 11 |  | Seg 1 | MF |
|  |  | 12 |  | Seg 2 | MF |
|  |  | 13 | SQZ-FC_QPD_B | Seg 3 | MF |
|  |  | 14 |  | Seg 4 | MF |
|  |  | 15 |  | Seg 1 | MF |
|  |  | 16 |  | Seg 2 | MF |

| Card | AA/AI conn. | ADC/DAC Chs. | Signal |
| --- | --- | --- | --- |
| DAC 0 | DB9_1 | 1 | SQZ-FC_BEAT_CTRL |
|  |  | 2 | SQZ-RLF_VCXO_CTRL |
|  |  | 3 | SQZ-ADF_VCXO_CTRL |
|  |  | 4 | SQZ-SEED-PZT |
|  | DB9_2 | 5 | OMC_TEST_DCPD_AB |
|  |  | 6 | OMC_TEST_DCPD_CD |
|  |  | 7 | OMC A PZT3 Drive (HV, bias) |
|  |  | 8 | OMC B PZT3 Drive (HV, bias) |
|  | DB9_3 | 9 | OMC A PZT1 Dither (LV) |
|  |  | 10 | OMC A PZT2 Drive (HV, servo) |
|  |  | 11 | OMC B PZT1 Dither (LV) |
|  |  | 12 | OMC B PZT2 Drive (HV, servo) |
|  | DB9_4 | 13 | ALS-C_REFL_DC_BIAS CM Summing module control |
|  |  | 14 | JAC_L |
|  |  | 15 | LSC-EXTRA_AO_3 |
|  |  | 16 | LSC-EXTRA_AO_4 |

# OMC I/O Chassis

The OMC I/O chassis is located in the SQZ-C1 rack.

| Card | AA/AI conn. | ADC/DAC Chns. | Signal |
| --- | --- | --- | --- |
| ADC 0 | DB9_1 | 1/5/9/13 | OMC-BIAS_DCPD_A |
|  |  | 2/6/10/14 | OMC-BIAS_DCPD_B |
|  |  | 3/7/11/15 | OMC-BIAS_DCPD_C |
|  |  | 4/8/12/16 | OMC-BIAS_DCPD_D |
|  | DB9_2 | 17/21/25/29 | Unused |
|  |  | 18/22/26/30 | Unused |
|  |  | 19/23/27/31 | Unused |
|  |  | 20/24/28 | Unused |
|  |  | 32 | DuoTone |

# Filter Cavity End Station

The front-end is shared between all systems interfacing HAM8 in the filter cavity end station: SEI, SUS, PEM and SQZ/ISC. Furthermore, PEM and ISC share an ADC. Only the first 12 channels are for ISC, the remainder for PEM.

| Card | AA/AI conn. | ADC/DAC Chs. | Signal |  |
| --- | --- | --- | --- | --- |
| ADC 1 | DB9_1 | 1-4 | SQZ-FC_TRANS_A | Quad Photodiodes |
|  | DB9_2 | 5-8 | SQZ_FC_TRANS_B |  |
|  | DB9_3 | 9 | Unused | 4 ch Generic PD interface: SQZT8 |
|  |  | 10 | Unused |  |
|  |  | 11 | Unused |  |
|  |  | 12 | Unused |  |

# End Stations

PEM and Photon Calibrator are using the first ADC card (ADC 0). This card implements the DuoTone readbacks. The first ISC card is ADC 1 (but the model calls it ADC0). The same is true for the DAC cards, where PEM and Photon Calibrator are using the first DAC.

| Card | AA/AI conn. | ADC/DAC Chs. | Signal |  |  |
| --- | --- | --- | --- | --- | --- |
| ADC 1 | DB9_1 | 1-4 | ASC-TRX(Y)_A | In-Vac Quad Photodiodes |  |
|  | DB9_2 | 5-8 | ASC-TRX(Y)_B |  |  |
|  | DB9_3 | 9-12 | ALS-X(Y)_QPD_A |  |  |
|  | DB9_4 | 13-16 | ALS-X(Y)_QPD_B |  |  |
|  | DB9_5 | 17 | ALS-X(Y)_WFS_A | Seg 1 | DC |
|  |  | 18 |  | Seg 2 | DC |
|  |  | 19 |  | Seg 3 | DC |
|  |  | 20 |  | Seg 4 | DC |
|  | DB9_6 | 21 | ALS-X(Y)_WFS_B | Seg 1 | DC |
|  |  | 22 |  | Seg 2 | DC |
|  |  | 23 |  | Seg 3 | DC |
|  |  | 24 |  | Seg 4 | DC |
|  | DB9_7 | 25 | LSC-X(Y) EXTRA_AI_1 |  |  |
|  |  | 26 | LSC-X(Y) EXTRA_AI_2 |  |  |
|  |  | 27 | LSC-X(Y) EXTRA_AI_3 |  |  |
|  |  | 28 | ALS-X(Y) REQSTATE_A (EtherCAT interface) |  |  |
|  | DB9_8 | 29 | Used by Electric Field Meter (PEM) |  |  |
|  |  | 30 | Used by Electric Field Meter (PEM) |  |  |
|  |  | 31 | Unused |  |  |
|  |  | 32 | Unused |  |  |

| Card | AA/AI conn. | ADC/DAC Chs. | Signal |  |
| --- | --- | --- | --- | --- |
| ADC 2 | DB9_1 | 1 | ALS-X(Y)_FIBR_SERVO_ERR CM Servo: ALS phase-locking, I monitor |  |
|  |  | 2 | ALS-X(Y)_FIBR_SERVO_CTRL CM Servo: ALS phase-locking, Fast monitor |  |
|  |  | 3 | ALS-X(Y)_FIBR_SERVO_SLOW CM Servo: ALS phase-locking, Slow monitor |  |
|  |  | 4 | Unused |  |
|  | DB9_2 | 5 | ALS-X(Y)_REFL_SERVO_ERR CM Servo: ALS PDH-locking, I monitor |  |
|  |  | 6 | ALS-X(Y)_REFL_SERVO_CTRL CM Servo: ALS PDH-locking, Fast monitor |  |
|  |  | 7 | ALS-X(Y)_FIBR_SERVO_SLOW CM Servo: ALS PDH-locking, Slow monitor |  |
|  |  | 8 | Unused |  |
|  | DB9_3 | 9 | ALS-X(Y)_REFL_B_LF PD1: DC from REFL power mon. | 4 ch. PD interface chassis: ALS Table |
|  |  | 10 | LSC-TRX(Y)_A_LF PD2: Red transmitted beam |  |
|  |  | 11 | ALS-X(Y)_LASER_GR_LF PD3: Green power monitor |  |
|  |  | 12 | ALS-X(Y)_FIBR_A_LF PD4: DC from BBPD |  |
|  | DB9_4 | 13 | ALS-X(Y)_LASER_IR_LF PD1: IR power monitor | 4 ch. Aux. signals concentrator 5: Field rack |
|  |  | 14 | ALS-X(Y)_FIBR_REJECTED_LF PD2: Rejected fiber power |  |
|  |  | 15 | ALS-X(Y)_FIBR_TRANS_LF PD3: Trans. fiber power |  |
|  |  | 16 | ALS-X(Y)_SPARE_B_LF PD4: unused |  |

| Card | AA/AI conn. | ADC/DAC Chs. |  | Signal |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ADC2 | DB9_5 |  | 17 | ALS-X(Y)_WFS_A | Seg 1 | Q-phase |
|  |  |  | 18 |  |  | I-phase |
|  |  |  | 19 |  | Seg 2 | Q-phase |
|  |  |  | 20 |  |  | I-phase |
|  | DB9_6 |  | 21 |  | Seg 3 | Q-phase |
|  |  |  | 22 |  |  | I-phase |
|  |  |  | 23 |  | Seg 4 | Q-phase |
|  |  |  | 24 |  |  | I-phase |
|  | DB9_7 |  | 25 | ALS-X(Y)_WFS_B | Seg 1 | Q-phase |
|  |  |  | 26 |  |  | I-phase |
|  |  |  | 27 |  | Seg 2 | Q-phase |
|  |  |  | 28 |  |  | I-phase |
|  | DB9_8 |  | 29 |  | Seg 3 | Q-phase |
|  |  |  | 30 |  |  | I-phase |
|  |  |  | 31 |  | Seg 4 | Q-phase |
|  |  |  | 32 |  |  | I-phase |

| Card | AA/AI conn. | ADC/DAC Chs. | Signal |
| --- | --- | --- | --- |
| DAC1 | DB9_1 | 1 | ALS-X(Y)_PZT1_PIT MCL PZT Tip-tilt 1: pitch |
|  |  | 2 | ALS-X(Y)_PZT1_YAW MCL PZT Tip-tilt 1: yaw |
|  |  | 3 | ALS-X(Y)_PZT2_PIT MCL PZT Tip-tilt 2: pitch |
|  |  | 4 | ALS-X(Y)_PZT2_YAW MCL PZT Tip-tilt 2: yaw |
|  | DB9_2 | 5 | LSC-X(Y) EXTRA_AO_1 |
|  |  | 6 | LSC-X(Y) EXTRA_AO_2 |
|  |  | 7 | LSC-X(Y) EXTRA_AO_3 |
|  |  | 8 | ALS-X(Y) REQSTATE_A (EtherCAT interface) |
|  | DB9_3 | 9 | ALS-X(Y)_ PZT3_PIT MCL PZT Tip-tilt 3 for WFS_A: pitch |
|  |  | 10 | ALS-X(Y)_PZT3_YAW MCL PZT Tip-tilt 3 for WFS_A: yaw |
|  |  | 11 | ALS-X(Y)_PZT4_PIT MCL PZT Tip-tilt 4 for WFS_B: pitch |
|  |  | 12 | ALS-X(Y)_PZT4_YAW MCL PZT Tip-tilt 4 for WFS_B: yaw |
|  | DB9_4 | 13 | Unused |
|  |  | 14 | Unused |
|  |  | 15 | Unused |
|  |  | 16 | Unused |

# ADC/DAC Channel Concentrator

## Variant 1

| Chas. | Conn. | ADC Chns. | Signal | Jumper |
| --- | --- | --- | --- | --- |
| 1 Front | DB9_1 IN | 1 | SQZ-OPO_SERVO_ERR, CM Servo, I monitor |  |
|  |  | 2 | SQZ-OPO_SERVO_CTRL, CM Servo, Fast monitor |  |
|  |  | 3 | SQZ-OPO_SERVO_SLOW, CM Servo, Slow monitor |  |
|  |  | 4 | Unused |  |
|  | DB9_2 IN | 5 | SQZ-SHG_SERVO_ERR, CM Servo, I monitor |  |
|  |  | 6 | SQZ-SHG_SERVO_CTRL, CM Servo, Fast monitor |  |
|  |  | 7 | SQZ-SHG_SERVO_SLOW, CM Servo, Slow monitor |  |
|  |  | 8 | Unused |  |
|  | DB9_3 IN | 9 | SQZ-LO_SERVO_ERR, CM Servo, I monitor |  |
|  |  | 10 | SQZ-LO_SERVO_CTRL, CM Servo, Fast monitor |  |
|  |  | 11 | SQZ-LO_SERVO_SLOW, CM Servo, Slow monitor |  |
|  |  | 12 | Unused |  |
|  | DB9_4 IN | 13 | SQZ-CLF_SERVO_ERR, CM Servo, I monitor |  |
|  |  | 14 | SQZ-CLF_SERVO_CTRL, CM Servo, Fast monitor |  |
|  |  | 15 | SQZ-CLF_SERVO_SLOW, CM Servo, Slow monitor |  |
|  |  | 16 | Unused |  |
|  | DB9_1 OUT | 1 | SQZ-OPO_SERVO_ERR, CM Servo, I monitor | IN1-1&6/OUT1-1&6 |
|  |  | 2 | SQZ-OPO_SERVO_CTRL, CM Servo, Fast monitor | IN1-2&7/OUT1-2&7 |
|  |  | 3 | SQZ-OPO_SERVO_SLOW, CM Servo, Slow monitor | IN1-3&8/OUT1-3&8 |
|  |  | 4 | SQZ-CLF_SERVO_ERR, CM Servo, I monitor | IN4-1&6/OUT1-4&9 |
|  | DB9_2 OUT | 5 | SQZ-SHG_SERVO_ERR, CM Servo, I monitor | IN2-1&6/OUT2-1&6 |
|  |  | 6 | SQZ-SHG_SERVO_CTRL, CM Servo, Fast monitor | IN2-2&7/OUT2-2&7 |
|  |  | 7 | SQZ-SHG_SERVO_SLOW, CM Servo, Slow monitor | IN2-3&8/OUT2-3&8 |
|  |  | 8 | SQZ-CLF_SERVO_CTRL, CM Servo, Fast monitor | IN4-2&7/OUT2-4&9 |
|  | DB9_3 OUT | 9 | SQZ-LO_SERVO_ERR, CM Servo, I monitor | IN3-1&6/OUT3-1&6 |
|  |  | 10 | SQZ-LO_SERVO_CTRL, CM Servo, Fast monitor | IN3-2&7/OUT3-2&7 |
|  |  | 11 | SQZ-LO_SERVO_SLOW, CM Servo, Slow monitor | IN3-3&8/OUT3-3&8 |
|  |  | 12 | SQZ-CLF_SERVO_SLOW, CM Servo, Slow monitor | IN4-3&8/OUT3-4&9 |
|  | DB9_4 | 13-16 | Unused |  |

| Chas. | Conn. | ADC Chns. | Signal | Jumper |
| --- | --- | --- | --- | --- |
| 1 Rear | DB9 IN1 | 1 | LSC-REFL_SERVO_ERR, CM Servo, I monitor |  |
|  |  | 2 | LSC-REFL_SERVO_CTRL, CM Servo, Fast |  |
|  |  | 3 | LSC-REFL_SERVO_SLOW, CM Servo, Slow |  |
|  |  | 4 | Unused |  |
|  | DB9 IN2 | 5 | IMC-I, IMC Servo, I monitor |  |
|  |  | 6 | IMC-F, IMC Servo, Fast monitor |  |
|  |  | 7 | IMC-L, IMC Servo, Slow monitor |  |
|  |  | 8 | Unused |  |
|  | DB9 IN3 | 9 | OMC_REFL_A DC |  |
|  |  | 10 | Available |  |
|  |  | 11 | Unused |  |
|  |  | 12 | Unused |  |
|  | DB9 IN4 | 13 | Unused |  |
|  |  | 14 | Unused |  |
|  |  | 15 | Unused |  |
|  |  | 16 | Unused |  |
|  | DB9 OUT1 | 1 | LSC-REFL_SERVO_ERR, CM Servo, I monitor | IN1-1&6/OUT1-1&6 |
|  |  | 2 | LSC-REFL_SERVO_CTRL, CM Servo, Fast | IN1-2&7/OUT1-2&7 |
|  |  | 3 | LSC-REFL_SERVO_SLOW, CM Servo, Slow | IN1-3&8/OUT1-3&8 |
|  |  | 4 | OMC_REFL_A DC | IN3-1&6/OUT1-4&9 |
|  | DB9 OUT2 | 5 | IMC-I, IMC Servo, I monitor | IN2-1&6/OUT2-1&6 |
|  |  | 6 | IMC-F, IMC Servo, Fast monitor | IN2-2&7/OUT2-2&7 |
|  |  | 7 | IMC-L, IMC Servo, Slow monitor | IN2-3&8/OUT2-3&8 |
|  |  | 8 | Available | IN3-2&7/OUT2-4&9 |
|  | DB9 OUT3 | 9 | Unused |  |
|  |  | 10 | Unused |  |
|  |  | 11 | Unused |  |
|  |  | 12 | Unused |  |
|  | DB9 OUT4 | 13 | Unused |  |
|  |  | 14 | Unused |  |
|  |  | 15 | Unused |  |
|  |  | 16 | Unused |  |

## Variant 2

| Chas. | Conn. | ADC Chns. | Signal | Jumper |
| --- | --- | --- | --- | --- |
| 2 Front | DB9_1 IN | 1 | SQZ-OPO_ISS_ERR |  |
|  |  | 2 | SQZ-OPO_ISS_CTRL |  |
|  |  | 3 | Unused |  |
|  |  | 4 | Unused |  |
|  | DB9_2 IN | 5 | SQZ-CLF_ISS_ERR |  |
|  |  | 6 | SQZ-CLF_ISS_CTRL |  |
|  |  | 7 | Unused |  |
|  |  | 8 | Unused |  |
|  | DB9_3 IN | 9 | ALS-C_REFL_DC_BIAS |  |
|  |  | 10 | LSC EXTRA_AO_2 |  |
|  |  | 11 | LSC-EXTRA_AO_3 |  |
|  |  | 12 | LSC-EXTRA_AO_4 |  |
|  | DB9_4 IN | 13 | OMC_TEST_ |  |
|  |  | 14 | Unused |  |
|  |  | 15 | Unused |  |
|  |  | 16 | Unused |  |
|  | DB9_1 OUT | 1 | SQZ-OPO_ISS_ERR | IN1-1&6/OUT1-1&6 |
|  |  | 2 | SQZ-OPO_ISS_CTRL | IN1-2&7/OUT1-2&7 |
|  |  | 3 | SQZ-CLF_ISS_ERR | IN2-1&6/OUT1-3&8 |
|  |  | 4 | SQZ-CLF_ISS_CTRL | IN2-2&7/OUT1-4&9 |
|  | DB9_2 OUT | 5 | Unused |  |
|  |  | 6 | Unused |  |
|  |  | 7 | Unused |  |
|  |  | 8 | Unused |  |
|  | DB9_3 OUT | 9 | ALS-C_REFL_DC_BIAS | IN3-1&6/OUT3-1&6 |
|  |  | 10 | LSC EXTRA_AO_2 | IN3-2&7/OUT3-2&7 |
|  |  | 11 | LSC-EXTRA_AO_3 | IN3-3&8/OUT3-3&8 |
|  |  | 12 |  | IN4-1&6/OUT3-4&9 |
|  | DB9_4 OUT | 13-16 | Unused |  |

| Chas. | Conn. | ADC Chns. | Signal | Jumper |
| --- | --- | --- | --- | --- |
| 2 Rear | DB9 IN1 | 1 | Unused |  |
|  |  | 2 | Unused |  |
|  |  | 3 | Unused |  |
|  |  | 4 | Unused |  |
|  | DB9 IN2 | 5 | Unused |  |
|  |  | 6 | Unused |  |
|  |  | 7 | Unused |  |
|  |  | 8 | Unused |  |
|  | DB9 IN3 | 9 | Unused |  |
|  |  | 10 | Unused |  |
|  |  | 11 | Unused |  |
|  |  | 12 | Unused |  |
|  | DB9 IN4 | 13 | Unused |  |
|  |  | 14 | Unused |  |
|  |  | 15 | Unused |  |
|  |  | 16 | Unused |  |
|  | DB9 OUT1 | 1 | Unused |  |
|  |  | 2 | Unused |  |
|  |  | 3 | Unused |  |
|  |  | 4 | Unused |  |
|  | DB9 OUT2 | 5 | Unused |  |
|  |  | 6 | Unused |  |
|  |  | 7 | Unused |  |
|  |  | 8 | Unused |  |
|  | DB9 OUT3 | 9 | Unused |  |
|  |  | 10 | Unused |  |
|  |  | 11 | Unused |  |
|  |  | 12 | Unused |  |
|  | DB9 OUT4 | 13 | Unused |  |
|  |  | 14 | Unused |  |
|  |  | 15 | Unused |  |
|  |  | 16 | Unused |  |

# Summary

Below is a summary of the number of I/O cards, unused channels, and available I/O slots for the ISC I/O Expansion Chassis. This assumes there are a total of 10 slots available in the I/O chassis for ADC and/or DAC cards. For the unused ADC channel column, the number in parentheses is the subset of these channels that are available Anti-Alias (AA) chassis on free DB9 connectors; the other channels are found on AA DB9 connectors which are only partially used.

| I/O Chassis | # ADC cards | # DAC cards | Unused ADC chans | Unused DAC chans | Available I/O slots |
| --- | --- | --- | --- | --- | --- |
| Vertex: ASC | 7 | 1 | 12 (8) | 6 (4) | 2 |
| Vertex: LSC | 4 | 1 | 11 (0) | 0 (0) | 5 |
| End X | 2 | 1 | 7 (4) | 4 (4) | 5 |
| End Y | 2 | 1 | 7 (4) | 4 (4) | 5 |
| Totals | 15 | 4 | 37 (16) | 14 (12) | 17 |

At the end stations, the I/O Expansion Chassis is shared with PEM. PEM has one ADC card, of which 14 channels are used, and one DAC card. The DAC card provides 8 channels (18 bit) of general purpose test outputs.  The ‘available slots’ number takes these PEM cards into account.