LIGO HANFORD OBSERVATORY

PO BOX 159

RICHLAND WA 99352

TEL: 509.372.8106

FAX: 509.372.8137

# MEMORANDUM

| DATE: TO: | January 29, 2025 Slow controls team |
| --- | --- |
| FROM: | Daniel Sigg |
| SUBJECT: | Modifications to the Beckhoff chassis |
| Refer to: | LIGO-E2000499-v5 |

Due to the elimination of the end station computers the remote access chassis in the end stations can be simplified. A set of terminals is removed including the bridge terminal that was used to communicate between computers.

Change 1: Remote Access End

Drawing: D1100619-v3
	Wiring diagram: E1100455-v3

Work: Combined the terminals into a single rail and
		remove terminals that are no longer needed
	Needed: 2x D1100095-v2 per end station (4 total/site)

Change 2: Remote Access Corner

Drawing: D1100618-v2

Work: Verify the position of the EL9400 terminal

The rotation stages are put on a separate rail that can be independently power cycled. As part of the A+ upgrade a picomotor controller for the PSL is added.

Change 3: PSL/IO/TCS Controls Chassis

Drawing: D1101114-v6
	Wiring diagram: E1100530-v8

Work: Separate rotation stage onto separate rails, add relay for power on/off

Add picomotor interface terminals for A+

Needed by rotation stage upgrade:

1x EK1122

1x EK1101

1x EL1094

1x EL2612

Needed by A+ upgrade:

1x D2000017-v1 (superseded by version 2)

2x D1100419-v3

1x EL9410

1x EL3102

1x EL1014

1x EL1872

1x EL2872

Controls and readbacks for the T-SAMS and the P-SAMS are needed for A+.

Change 4: PSL/IO/TCS Controls Chassis

Drawing: D1101114-v7

Wiring diagram: E1100530-v9

Work: Add terminals for a new middle rail for the P-SAMS

Add terminals to the end of the right rail for the T-SAMS

Needed for the new middle rail:

1x D2000017-v2

2x D0902567-v1

1x EK1100

4x EL3104

1x EL9190

1x EL9011

Needed for the right rail upgrade:

2x D0902568-v1

1x EL9410

2x EL3104

2x EL3692

1x EL4132

Additional picomotor controller interfaces are added to the corner chassis 2 for A+.

Change 5: EtherCAT Corner 2

Drawing: D1100680-v3
	Wiring diagram: E1101126-v3

Work: Add 3 picomotor controller interfaces

Needed by A+ upgrade:

1x D1102268-v2

1x EL9410

3x EL3102

3x EL1014

3x EL1872

3x EL2872

1x D1100419-v3

The EtherCAT chassis near HAM8 may require picomotor controllers as well.

Needed by A+ upgrade:

2x D1100419-v3

Two sets of controls for the AM modulated AOM amplifiers are added to the EtherCAT chassis squeezer 3 for A+. Also, added are interfaces for the DC PD concentrator 9 and an additional auxiliary concentrator 1.

Change 6: EtherCAT Squeezer 3

Drawing: D1600509-v4

Wiring diagram: E1600386-v4

Work: Add controls for an AM modulated AOM amplifiers to the middle rail

Add a new right rail for another AM modulated AOM amplifiers, and the two new concentrators.

Needed by A+ upgrade for the middle rail:

1x D1700036-v2

1x D0902569-v1

1x Omron S82S-7305

1x EL9410

1x EL3104

1x EL4134

1x EL1124

4x EL2124

Needed by A+ upgrade for the right rail:

3x D0902569-v1

3x Omron S82S-7305

1x EK1100

2x EL9410

4x EL3104

1x EL3102

2x EL4134

2x EL1124

8x EL2124

A new EtherCAT chassis squeezer 4 is needed for the additional controls of the filter cavity.

Change 7: EtherCAT Squeezer 4

Drawing: D2000504-v1

Wiring diagram: E2100047-v1

Work: Install new chassis

Maye require additional +24V power supply.

Change 8: EtherCAT Corner 6

Drawing: D1300745-v6

Wiring diagram: E1300689-v8

Work: Add controls for the 3.125 MHz squeezer pickoff and the DCPD relays

Needed by A+ upgrade for the middle rail:

1x EL9410 (replace M9, reuse EL9190)

Needed by A+ upgrade for the right rail:

1x D1500088-v2 (rear panel)

1x D0902567-v1 (DB25M adapter)

1x D0902569-v1 (DB37M adapter)

1x Omron S82S-7305

1x EL9410 (replace R10, reuse EL9190)

1x EL3104

1x EL1124

2x EL2004

A new EtherCAT chassis to the filter cavity end station.

Change 9: EtherCAT Filter Cavity End

Drawing: D2200006-v1

Wiring diagram: E2200004-v1

Work: Install new chassis in filter cavity end station

Requires installation of an EtherCAT media converter for single-mode fiber optic, CU1521-0010, in the MSR.

A new EtherCAT chassis squeezer 5 is needed for the squeezer PMC.

Change 10: EtherCAT Squeezer 5

Drawing: D2300323-v1

Wiring diagram: E2300329-v1

Work: Install new chassis.

Double up the EL3692 resistance terminals.

Change 11: EtherCAT Squeezer 2, 3, and 4

Drawing: D1600507-v2, D1600509-v5, and D2000504-v2

Wiring diagram: E1600384-v4, E1600386-v5, and

Work: Double up the EL3692 resistance terminal.

1x EL3692 (replace R11 in D1600507)

2x EL3692 (replace L20/L22 in D1600509)

2x EL3692 (replace M14/M23 in D2000504)

Make sure that wires for digital step attenuator in PLL integration have been added (D2000504).

Add support for BHD.

Change 12: EtherCAT Corner 4 (implement together with change 14)

Drawing: D1101266-v4

Wiring diagram: E1101222-v5

Work: Add readbacks for another WFS and LSC demodulator concentrators

Needed by BHD upgrade for the left rail:

2x EL9410 (replace M5 and add M15)

Needed by BHD upgrade for the middle rail:

2x EL9410 (replace M5 and add M15, reuse EL9190)

4x EL3104

Needed by BHD upgrade for the right rail:

1x EL9410

6x EL3104

1x EL1124

1x D2400266-v1 (rear panel)

1x D0902557-v1 (blank rear panel)

2x D0902569-v1 (DB37M adapter)

1x Omron S82S-7305 or Mean Well DDR-15G-5

Change 13: EtherCAT Corner 6

Drawing: D1300745-v6

Wiring diagram: E1300689-v9

Work: Update wiring to incorporate second fast shutter and DCPD whitening

Needed by BHD upgrade for the middle rail:

No new terminals, just wire changes for fast shutter interface.

Add grey wires as indicated in the wiring diagram between terminal M2 and adapter in slot 8.

Add support for JAC.

Change 14: EtherCAT Corner 4 (implement together with change 12)

Drawing: D1101266-v4

Wiring diagram: E1101222-v5

Work: Add shutter controls

Needed by JAC upgrade for the middle rail:

1x EL9410
1x EL1094

1x EL2008

1x EL3104
1x EL4134

Change 15: EtherCAT Corner 5

Drawing: D1200132-v4

Wiring diagram: E1200077-v4

Work: Add resistance terminals to read JAC temperature

Needed by A+ upgrade for the left rail:

1x EL9410 (replace L5)

Needed by JAC upgrade for the middle rail:

1x EL9410 (replace M8)

2x EL3692